Global Check Setup For Penny Lane POS

- Launch Penny Lane Manager, select Setup / Advanced Setup / Terminal

Credit/Debit page.  Set up fields as follows :

Modem Configuration :

Modem Port :		specify the port that your modem is installed on

Baud :			9600

Parity :			EVEN

Data Bits :		7

Stop Bits :		1

Check Authorization :

Auth Service :		Global Check

Merchant ID :		assigned by Global Check

Phone# :		provided by Global Check

Default State Code :	specify the merchant’s home State Code ( eg. Kansas = 40 )

Check Reader page.  Set up fields as follows :

Port :		  	specify the port that the Magtek Mini MICR is attached to

Baud :			1200

Parity :			EVEN

Data Bits :		7

Stop Bits :		1

- In Penny Lane Manager select Setup / Advanced Setup / Store

Tender Authorization page.  Set up fields as follows :

Auth Service :	set to AUTH/EDC for Check tender type field