Penny Lane POS Windows 7 Special Setup Instructions

This document outlines the various steps that must be taken in order to make Penny Lane POS or Penny Lane Retail Enterprise run properly on a Windows 7 machine.

- Environment Variables ( TEMP & TMP )

In the past, we have routinely set these variable ( both System and User copies of the TEMP 	& TMP variables ) to C:\WINDOWS\TEMP.  With Windows 7 it is necessary to go outside 	of the C:\WINDOWS folder, so create a folder C:\TEMP and set the TEMP & TMP System 	and User variables to this folder.  Be sure to set all users to have the “Full Control” security 	level to this folder.

- Borland Database Engine Privileges

The “node” in the Windows registry which pertains to the Borland Database Engine must 	have “Full Control” access privilege for the login user.  With previous versions of Windows 	the Borland Database Engine settings are placed inside the HKEY_LOCAL_MACHINE | 	SOFTWARE | Borland node.  This is occasionally true when Penny Lane POS is installed 	on a Windows 7 machine, but it is far more likely to be installed inside 	HKEY_LOCAL_MACHINE | SOFTWARE | Wow6432Node.  It is this node which must 	be given the “Full Control” access privilege.  To do this right click the node and select 	“Permissions”.  In the “Group Or User Names” section click on Users.  In the “Permissions 	For Users” section check the “Allow” box for both “Full Control” and “Read”, then click 	OK to save those settings.  After doing that, you should notice that all nodes below that node 	will inherit those access privileges.

- Borland Database Engine Settings

If the Borland Database Engine is installed under the “Wow6432Node” node in the 	Windows registry, its settings can only be modified directly in the registry using the 	REGEDIT program.

Inside “Wow6432Node | Borland |Database Engine | Settings | DRIVERS |DBASE | 	INIT” the recommended settings are as follows :

LANGDRIVER = DBWINUS0

Inside “Wow6432Node | Borland |Database Engine | Settings | SYSTEM | INIT” the 	recommended settings are as follows :

DEFAULT DRIVER = DBASE

LANGDRIVER = DBWINUS0

LOCAL SHARE = TRUE

MAXBUFSIZE = 9600

MAXFILEHANDLES = 256

MEMSIZE = 205

MINBUFSIZE = 9600

SHAREDMEMSIZE = 1024

In addition, it is usually necessary to put something into the SHAREDMEMLOCATION 	field in order to get rid of the “Error initializing the Borland Database Engine ( Error $2501 )” 	error.  With this one, it’s a matter of trial and error.  Here are some possible values to try :

1000

2000

3000

4000

5000

6000

7000

8000

5BDE	 we’ve had some success using this value.

- Setting Access Privileges on the Backoffice “Server” Computer.

  - Make sure all computers have the same Workgroup name.

  - On the Backoffice machine go into Control Panel | Network & Internet | Network & Sharing Centre, select Change Advanced Sharing Settings.  Set everything to “ON”, except Password Protection, which should be set to “OFF”.

  - Choose which folders to share.  You can either share the C:\PLM folder or share the whole C: drive.  In Windows Explorer ( on the backoffice machine ) right click the folder ( or drive ) and select Properties.  Select the Sharing tab, click on Advanced Sharing, then tick the “Share this folder” checkbox, then click Apply, then OK, then Close.

  - In Windows Explorer ( on the backoffice machine ) right click the folder ( or drive ), and select Properties.  Select the Security tab, click Edit, then click Add, type “everyone”, click OK, tick all permissions ( Full Control, etc. ), click Apply, click OK.  When you do this you will get some errors popping ( especially if you are doing the whole C: drive ).  Just click Continue to get past these errors.  It’s just that there are certains folders/files that W7 cannot give you full permissions to.

At this point your XP machines will have proper access to read and write to the C:\PLM folder on the backoffice computer.  You should use the Universal Naming Convention when referencing folders on the backoffice.  So, assuming the Computer Name of the backoffice computer is BACKOFFICE, and you have shared the whole C: drive and the Share Name is simply C, then in the Store Setup | Terminal Dirs you should have something like :

Combine/Poll : 	\\BACKOFFICE\C\PLM\POLL

POS1 : 		\\BACKOFFICE\C\PLM\TERM1

POS2 : 		\\BACKOFFICE\C\PLM\TERM2

Etc.

If you shared only the C:\PLM folder ( and the Share Name is PLM ) then you should have :

Combine/Poll : 	\\BACKOFFICE\PLM\POLL

POS1 : 		\\BACKOFFICE\PLM\TERM1

POS2 : 		\\BACKOFFICE\PLM\TERM2

Etc.

In Penny Lane Shop Floor, in the System Setup you should be referring to the backoffice computer as follows

If you had share the whole C: drive :

Network Program Directory : \\BACKOFFICE\C\PLM\PROGRAMS

Network Program Directory : \\BACKOFFICE\C\PLM\DATA

If you had share the just the C:\PLM folder :

Network Program Directory : \\BACKOFFICE\PLM\PROGRAMS

Network Program Directory : \\BACKOFFICE\PLM\DATA

NOTE : It would be a good idea to set the privileges the same way on each POS terminal.

- Change User Account Control Settings

Certain programs in Penny Lane POS or Penny Lane Retail Enterprise cause Windows 7 to 	pop up the security prompt “Do you wish to allow this program to make changes to your 	hard drive?”.  To defeat this go into “Control Panel”, select “User Accounts”, then select 	“Change User Account Control Settings”.  Slide the control down to the “Never Notify” 	setting, then click OK to save that setting.